SMART FIRE DETECTION SYSTEM

Wi-Fi Connected Fire Alarm with WhatsApp Alerts

Technical Documentation  |  Version 1.0

Platform: ESP32 ¦ Language: Arduino C++ ¦ Connectivity: Wi-Fi 802.11 b/g/n ¦ Alert: WhatsApp (CallMeBot API)

# 1. Project Overview

The Smart Fire Detection System is an IoT-based solution built on the ESP32 microcontroller. It continuously monitors environmental conditions using a digital fire/flame sensor and, upon detecting fire, immediately triggers a local buzzer alarm and dispatches a real-time WhatsApp notification to a designated phone number through the CallMeBot API.

## 1.1  Objectives

- Provide real-time fire detection using a digital flame sensor.

- Alert users locally via an audible buzzer.

- Send instant WhatsApp messages to a registered phone number.

- Confirm system readiness on startup with an initial WhatsApp notification.

## 1.2  Key Features

| Real-Time Detection | Continuous polling of flame sensor every loop cycle |
| --- | --- |
| Dual Alert Mechanism | Buzzer alarm + WhatsApp push notification |
| Startup Confirmation | "Your system is ready!" message sent on boot |
| HTTP API Integration | Uses CallMeBot WhatsApp API over HTTPS |
| Serial Debugging | 115200 baud serial output for diagnostics |
| Wi-Fi Connectivity | Automatic reconnection with status feedback |

# 2. Hardware Components

| Microcontroller | ESP32 (dual-core, 240 MHz, built-in Wi-Fi & Bluetooth) |
| --- | --- |
| Flame Sensor | Digital IR Flame Sensor Module (DO output) |
| Buzzer | Active Buzzer Module (5V compatible) |
| Power Supply | USB 5V via ESP32 dev board |
| Connectivity | Wi-Fi 802.11 b/g/n (2.4 GHz) |

## 2.1  Pin Assignment

| GPIO 32 | Flame Sensor (INPUT) — digital reading from DO pin |
| --- | --- |
| GPIO 33 | Buzzer (OUTPUT) — HIGH = alarm ON, LOW = alarm OFF |
| GND | Common ground for all modules |
| 3.3V / 5V | Power supply for sensor and buzzer modules |

## 2.2  Wiring Diagram

Flame Sensor DO → GPIO 32 ¦ Flame Sensor VCC → 3.3V ¦ Flame Sensor GND → GNDBuzzer (+) → GPIO 33 ¦ Buzzer (-) → GND

# 3. Software Architecture

## 3.1  Libraries Used

| WiFi.h | ESP32 built-in — manages Wi-Fi connection |
| --- | --- |
| HTTPClient.h | ESP32 built-in — sends HTTP GET/POST requests |
| UrlEncode.h | Third-party — URL-encodes WhatsApp message strings |

## 3.2  Configuration Constants

#define Buzzer  33          // GPIO pin for buzzer

#define Sensor  32          // GPIO pin for flame sensor

const char* ssid     = "YourSSID";       // Wi-Fi network name

const char* password = "YourPassword";   // Wi-Fi password

String phoneNumber = "+201XXXXXXXXX";    // WhatsApp number (E.164 format)

String apiKey      = "XXXXXXX";          // CallMeBot API key

## 3.3  Program Flow

### setup() — Runs once on boot

- Initialize Serial at 115200 baud for debugging.

- Configure GPIO 33 as OUTPUT (Buzzer) and GPIO 32 as INPUT (Sensor).

- Connect to Wi-Fi; print dots until connected, then print assigned IP address.

- Call sendAlert() to send the startup confirmation message to WhatsApp.

### loop() — Runs continuously

- Read digital value from Sensor (GPIO 32).

- If value == 1 (fire detected): turn Buzzer ON and send WhatsApp alert.

- If value == 0 (no fire): keep Buzzer OFF.

### sendAlert(String message)

- Build the CallMeBot API URL with phone, apikey, and URL-encoded message.

- Initialize HTTPClient and set Content-Type header.

- Send HTTP POST request; log success (200) or error code to Serial.

- Release HTTP resources with http.end().

## 3.4  Full Source Code

#include <WiFi.h>

#include <HTTPClient.h>

#include <UrlEncode.h>

#define Buzzer 33

#define Sensor 32

const char* ssid     = "YOUR_SSID";

const char* password = "YOUR_PASSWORD";

String phoneNumber = "+201XXXXXXXXX";

String apiKey      = "XXXXXXX";

void setup() {

Serial.begin(115200);

pinMode(Buzzer, OUTPUT);

pinMode(Sensor, INPUT);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); }

Serial.println("\nConnected. IP: " + WiFi.localIP().toString());

sendAlert("Your system is ready!");

}

void loop() {

bool sensorValue = digitalRead(Sensor);

if (sensorValue == 1) {

digitalWrite(Buzzer, HIGH);

sendAlert("Warning! Fire Detected...");

} else {

digitalWrite(Buzzer, LOW);

}

}

void sendAlert(String message) {

String url = "https://api.callmebot.com/whatsapp.php?phone="

+ phoneNumber + "&apikey=" + apiKey

+ "&text=" + urlEncode(message);

HTTPClient http;

http.begin(url);

http.addHeader("Content-Type","application/x-www-form-urlencoded");

int code = http.POST(url);

Serial.println(code == 200 ? "Sent!" : "Error: " + String(code));

http.end();

}

# 4. CallMeBot WhatsApp API

The project uses the free CallMeBot API to deliver WhatsApp messages without requiring a Business API account. Communication is via a standard HTTPS GET/POST request.

## 4.1  API Endpoint

https://api.callmebot.com/whatsapp.php?phone={number}&apikey={key}&text={message}

## 4.2  Parameters

| phone | Recipient phone number in E.164 format (e.g. +201019972777) |
| --- | --- |
| apikey | Personal API key obtained from the CallMeBot WhatsApp bot |
| text | URL-encoded message string (handled by UrlEncode library) |

## 4.3  HTTP Response Codes

| 200 OK | Message delivered successfully |
| --- | --- |
| 400 Bad Request | Missing or malformed parameters |
| 401 Unauthorized | Invalid or unregistered API key |
| 429 Too Many Requests | Rate limit exceeded; reduce sending frequency |

## 4.4  Obtaining Your API Key

- Open WhatsApp and add +34 644 59 21 83 to your contacts.

- Send the message: I allow callmebot to send me messages

- The bot will reply with your personal API key within seconds.

- Use that key in the apiKey variable of the sketch.

⚠️ Security Note: Never share your API key publicly. Store credentials in a separate config file or use environment variables for production deployments.

# 5. Setup & Deployment Guide

## Step 1 — Install Arduino IDE & ESP32 Board Package

- Download Arduino IDE from arduino.cc

- Go to File > Preferences and add the ESP32 board URL:

  - https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Open Tools > Board Manager, search "esp32", and install the Espressif package.

## Step 2 — Install Required Library

- Open Sketch > Include Library > Manage Libraries.

- Search for "UrlEncode" and install the library by Masayuki Sugahara.

## Step 3 — Configure Your Credentials

- Replace YOUR_SSID and YOUR_PASSWORD with your Wi-Fi credentials.

- Replace the phone number and API key with your own values.

## Step 4 — Upload the Sketch

- Connect ESP32 to your PC via USB.

- Select the correct board (e.g. "ESP32 Dev Module") and COM port.

- Click Upload and wait for the "Done uploading" message.

## Step 5 — Verify Operation

- Open Serial Monitor at 115200 baud.

- Confirm the ESP32 connects to Wi-Fi and prints the IP address.

- Check WhatsApp for the "Your system is ready!" confirmation message.

- Test by bringing a flame near the sensor; the buzzer should sound and a WhatsApp alert should arrive.

# 6. Troubleshooting

| Wi-Fi not connecting | Verify SSID/password; ensure 2.4 GHz band; move closer to router |
| --- | --- |
| No WhatsApp message | Re-register API key with CallMeBot bot; check phone number format (+country code) |
| HTTP error 429 | Too many alerts sent; add delay() after sendAlert() in the loop |
| Buzzer always ON | Check sensor wiring and sensor sensitivity trim potentiometer |
| Buzzer never ON | Test sensor output with Serial.println(sensorValue); check DO pin level |
| Upload fails | Hold BOOT button on ESP32 during upload; check driver installation |

# 7. Known Limitations & Future Improvements

## 7.1  Current Limitations

- The loop continuously sends WhatsApp messages while fire is detected, which may hit API rate limits (429 errors).

- No Wi-Fi reconnection logic if the connection drops during operation.

- Credentials (SSID, password, API key) are hardcoded in the source file.

- Single sensor only; no redundancy or multi-zone detection.

## 7.2  Suggested Improvements

- Add a flag variable (alertSent) to send only one WhatsApp message per fire event.

- Implement Wi-Fi reconnection logic using WiFi.reconnect() in the loop.

- Store credentials in EEPROM or use Wi-Fi provisioning (SmartConfig / BLE).

- Add an OLED display to show real-time sensor status locally.

- Integrate a temperature/humidity sensor (DHT22) for richer alert data.

- Add Telegram or email as backup notification channels.

- Implement MQTT for centralized IoT dashboard monitoring.

## 7.3  Recommended Code Fix (Rate Limiting)

bool alertSent = false;

void loop() {

bool sensorValue = digitalRead(Sensor);

if (sensorValue == 1 && !alertSent) {

digitalWrite(Buzzer, HIGH);

sendAlert("Warning! Fire Detected...");

alertSent = true;

} else if (sensorValue == 0) {

digitalWrite(Buzzer, LOW);

alertSent = false;   // Reset when fire clears

}

}

# 8. Technology Stack Summary

| Microcontroller | ESP32 (Espressif Systems) |
| --- | --- |
| Programming Language | Arduino C++ (C++11) |
| IDE | Arduino IDE 2.x |
| Networking Protocol | HTTPS over Wi-Fi (TLS 1.2) |
| Notification API | CallMeBot WhatsApp API (REST) |
| Libraries | WiFi.h, HTTPClient.h, UrlEncode.h |
| Baud Rate (Serial) | 115200 |
| Sensor Interface | Digital GPIO (HIGH = fire, LOW = safe) |
| Actuator Interface | Digital GPIO (HIGH = buzzer ON) |
| Document Version | 1.0 |
| Author | Smart Systems Project |

For questions or contributions, please refer to the source code and this documentation. Always test the system in a controlled environment before deploying in a real scenario.